Objective

- Securing freight on vehicles to ensure minimal damage occurs during transit

- Ensuring freight is transported safely and preventing damage or loss of items during transit

- Freight securing equipment must always be available when loading freight for deliveries or collections

Securing loads before departure

- Types of Fastening equipment

- Cargo Nets

- Belts and Rachets

- Chains and Satangs

- Tarpaulin

- Corner plates

Proper loading and securing of freight

- Items are properly packed on a pallet or stand.

- Supplier to ensure all items are properly plastic wrapped and packed to prevent movement during transit.

Items clearly marked with Destination and Order Number + Waybill Number

- Customer to ensure all items are clearly marked or labelled with the following information:

- Customer details

- Waybill number

- Consignee details

- Number of parcels

- Drivers must ensure all items are marked as above before loading.

How to properly secure loads

- Cylinders must be on V-Block or a stand.

- All round objects must be secured in a way to reduce the risk of rolling and falling off the truck.

- Cylinders can be tied down with either belts and ratches or chains and satangs.

Spindles

- Spindles must be secured on a stand.

- Ensure whilst loading that the spindle is not moving.

- Secure with 1 chain and satang as seen on the Pictures.

Alternator, Traction motors and Tracks

- Alternators, Blower motors and Traction motors to be secured on a steel pallet with chains and satangs.

- Tracks to be loaded on pallets or dunnage wood and secured with chains and satangs.

Dump, Hoist and Suspension Cylinders

- Cylinders are secured to an upright stand.

- Ensure that all the bolts are properly fastened before loading.

- Use chains and ratchets to secure the stands to the truck.

Final Drives

- Final drives are bolted to a stand.

- Final drives and stand to be secured with chains and satangs to prevent movement whilst in transit.

Engines and transmissions

- Engines and transmissions are secured to the stand stands.

- No engine or transmission can be loaded or accepted if not secures on a stand.

- Use chains and Ratchets to sure this to the truck.

- Customer to ensure engines and transmissions are secured on stands before loading.

Huge Items to be secured with chains

- All heavy items to be secured with chains and satangs to prevent movement during transit.

- Drivers are responsible to secure loads before departure.

Should Cargo not be secured as per above:

- Driver to report immediately to Direct Line Manager

- Driver to take photos of the Cargo (if possible)

- Line Manager / Account Executive to contact the customer.

- Driver to wait for Line Manager instruction.

Securing loads before departure:

- Secure cargo on vehicles to prevent in transit damage.

- Cargo securing equipment must always be used when loading cargo.

  - Cargo securing equipment:

- Types of Fastening equipment

- Cargo Nets

- Belts and Ratchets

- Chains and Satangs

- Tarpaulin

- Corner plates