	[image: image1.png]Express




	      ZAF - Secure Cargo WI
Owner/Department: Operations




1. Purpose (Objectives)

1.1 The purpose of this document is to provide instructions for securing cargo prior to transportation
2. Scope

2.1 This procedure applies to all applicable operational transportation activities.
3. Responsibilities

3.1 All applicable employees transporting loads.
4. Work Instruction
4.1 Properly Secure Loads
· Cylinders must be on V-Block or a stand.

· All round objects must be secured in a way to reduce the risk of rolling and falling off the truck.

· Cylinders can be tied down with either belts and ratches or chains.
4.2 Dump, Hoist and Suspension Cylinders
· Cylinders are secured to an upright stand.

· Ensure that all the bolts are properly fastened before loading.

· Use chains and ratchets to secure the stands to the truck.
4.3 Final Drives

· Final drives are bolted to a stand.
· Final drives and stand to be secured with chains to prevent movement whilst in transit.
4.4 Spindles

· Spindles must be secured on a stand.

· Ensure whilst loading that the spindle is not moving.

· Secure with 1 chain as seen on the Pictures.
4.5 Engines and transmissions:
· Engines and transmissions are secured to the stand stands.

· No engine or transmission can be loaded or accepted if not secures on a stand. 

· Use chains and Ratchets to secure this to the truck.

· Customer to ensure engines and transmissions are secured on stands before loading.
4.6 Alternator, Traction motors and Tracks:
· Alternators, Blower motors and Traction motors to be secured on a steel pallet with chains.

· Tracks to be loaded on pallets or dunnage wood and secured with chains 
4.7 Boxes:
· Boxes with arrows - make sure the arrows always pointing up

· Heavy freight / pallets must be loaded at the bottom and light ones at the top

· Secure loose freight with a cargo net
4.8 Huge Items to be secured with chains:
· All heavy items to be secured with chains to prevent movement during transit.
· Drivers are responsible to secure loads before departure.
Page 1 of 2 
Uncontrolled copy if printed 


[image: image1.png]